Musée des Confluences

Location: Lyon, France

Dates: 2001/2010 - 2014

Client: Conseil Géneral du Rhône, Lyon, France

Client Representative: SERL, Lyon, France

“The clear, readable shape of the Crystal stands for the world in which we move each day. The Cloud, by contrast, holds the knowledge of the future. What is known and what is to be explored are understood in the Musée des Confluences through a spatial experimental design meant to stimulate public curiosity.” – Wolf dPrix

Project Description

Right from the 2001 international competition for a natural history museum in Lyon, the museum was envisioned as a "medium for the transfer of knowledge" and not as a showroom for products.

The building ground of the museum is located on a peninsula that was artificially extended 100 years ago and situated at the confluence of the Rhône and Saône rivers. Even though it was apparent that this site would be a difficult one (536 piles had to be securely driven 30 meters into the ground), it was clear that this location would be very important for urban design. The building should serve as a distinctive beacon and entrance for the visitors approaching from the South, as well as a starting point for urban development.

In order to build a museum of knowledge, a complex new form had to be developed as an iconic gateway. A building that truly stands out can only come into being through shapes resulting from new geometries. It was important to the concept that the flow of

visitors arriving from the city to the Pointe du Confluent should not be impeded by a building. The idea was therefore to develop an openly traversable building that would be floating in part only on supports, in order to create a public space underneath.

Essentially, the building consists of three parts. Situated on a slightly raised base (due to the high groundwater), two auditoriums (for 327 and 122 persons, respectively) and workspaces, which will also be used for training purposes for the surrounding schools, will be located next to storage and workshops for the production of exhibitions.

The entrance building, the so-called Crystal, is openly traversable, and has vertical access to the exhibition spaces. The so-called Espace liant, a connecting path, can be reached by an escalator, a staircase, and a spiral ramp. Left and right of this path are arranged the individual exhibition halls (one of them two-level), and at the end is a view of the confluence of two rivers, the Pointe du Confluent. The steel structure, conceived as bridge construction, made it possible to develop all of the exhibition halls without support. The administration rooms are located above the exhibition spaces.

In the Plaza below this highly raised, almost flying component – the showrooms are broadly cantilevered in parts – the lit wave pattern of the surface of a small lake is reflected on the underside of the building. A brasserie emphasizes the public nature of this place. A freely accessible terrace café is located on the top floor.

In the entrance building, a drop-shaped construction serves as a supporting structure. Its form was developed out of the turbulent flow created by the confluence of the two streams. This gravity well reduces the weight of the entire steel structure of the entrance building by a third.

Musée des Confluences has yearly approx. 500.000 visitors.

Technical Description

Environmental concept

The foyer (Crystal) is a naturally ventilated space. The supply air enters via glazed ventilation flaps inside the east facade while the exhaust air exits via the roof area. It is, therefore, unnecessary to use a traditional air conditioning system. Only the main access areas and workspaces are microclimatic units whose comfort is ensured through local heating and cooling systems. The floors are cooled via groundwater. This will result in significant energy savings for the museum’s foyer in the long term.

In terms of thermal insulation, the facades of the exhibition area (Cloud) are characterized by an extremely efficient building shell. All of the main access areas are illuminated naturally (not much artificial light); the water supply of the sanitation areas is provided by groundwater. A photovoltaic system is installed on the roof.

Materials

Base:

The base is designed as a reinforced concrete structure with exposed concrete walls.

Foyer / “Crystal”:

A tubular lattice with a rectangular profile of 400 x 200 mm forms the primary support of the Crystal. It rests on the concrete structures of the base and the main supporting structure of the Cloud. The center of the Crystal is occupied by the Puits de gravité, which continues the primary support and plays the role of a major support element that diverts all forces. It reduces the weight of the entry building’s entire steel construction by a third.

The secondary support structure carries the large glass panels. It consists of steel tubing arranged in the grid of the glass panels. It is connected to the primary support structure via struts made of tubes that are screwed to panels welded to the primary supports.

Blinds between the primary and secondary supporting structures protect the most sun-exposed areas and also reduce noise.

The glass consists of single-glazed panels with extra-clear glass.

A large amount of glazed, openable windows in the different areas of the Crystal provide natural ventilation. They can be opened through a motor. Additional deeper and higher openings can provide smoke extraction in case of fire. The four different entry sequences consist of large glass doors. The main entrance has a large canopy that is connected to the primary support structure and clad with metal sheets, like the Cloud.

Exhibition area / “Cloud”:

The Cloud’s structural system resembles a bridge structure. It rests on 12 concrete supports and three concrete towers that contain the emergency stairs and shafts. Room-high steel frameworks form the walls of the black boxes for permanent and temporary exhibitions.

The outer skin of the Cloud consists of 3-mm stainless steel plates that have been blasted with glass beads. This special surface treatment results in a gentle reflection of the light and colors of the surrounding area.

Project Data:

Site Area:                                     20,975 m²

Gross Floor Area:                       46,476 m²

Net Floor Area:                            26,700 m²

Footprint:                                     9,300 m²

Volumes

Total: 195,206 m³

Base (incl. Brasserie): 59,436 m³

Crystal / Foyer: 25,770 m³

Cloud / Exhibition space: 110,000 m³

Dimensions

Length: 190 m

Width: 90 m

Height: 41 m

Base

2 auditoriums with 327 and 122 seats

working rooms for classes, conference and meeting rooms,

storage, workshops, HVACR, logistics, group entrance

brasserie on top (publicly accessible)

Crystal / Foyer

Main entrance, foyer, library / shop

Cloud / Exhibition space

9 Exhibition rooms

Level 1: temporary exhibitions (5 rooms) + public ateliers for workshops

Level 2: permanent collections (4 rooms) + public ateliers for workshops

Level 3: Administration

Rooftop café (publicly accessible)

Timeline

Competition: 2001

Preliminary design / Design development: 2002-2004

1. Tender / Dialog Competitive: 2004-2006

Construction phase 1 (pile foundation): 2006-2007

Stop of construction: 2007-2009

2. Tender / Execution design: 2009-2010

Construction phase 2: 2010-2014

Completion: 12/2014

Project Team

Planning: COOP HIMMELB(L)AU Wolf D. Prix & Partner ZT GmbH

Design Principal: Wolf dPrix

Project Partner: Markus Prossnigg

Design Architect: Tom Wiscombe

Project Architects: Mona Bayr, Angus Schoenberger

Project Coordination: Thomas Margaretha, Peter Grell

Project Team Vienna: Christopher Beccone, Guy Bébié, Lorenz Bürgi, Wolfgang Fiel, Kai Hellat, Robert Haranza, Alex Jackson, Georg Kolmayr, Daniel Kerbler, Lucas Kulnig, Andreas Mieling, Marianna Milioni, Daniel Moral, Jutta Schädler, Andrea

Schöning, Mario Schwary, Markus Schwarz, Oliver Tessmann, Dionicio Valdez, Philipp Vogt, Markus Wings, Christoph Ziegler

Project Team Lyon: Patrick Lhomme, Francois Texier, Philippe Folliasson, Etienne Champenois, Alexandru Gheorghe, Niels Hiller, Emanuele Iacono, Pierre-Yves Six

Consultants:

Local Architects: Patriarche & Co, Chambéry/Lyon, France (planning);

Tabula Rasa / Grégory Perrin, Lyon, France (execution); Chabanne & Partenaires, Lyon, France (project management)

Construction Survey Lyon: Debray Ingénierie, Caluire et Cuire, France

Costs: Mazet & Associés, Paris, France; CUBIC, Jean Luc Minjard, Lyon, France

Structural Engineering: B+G Ingenieure, Bollinger und Grohmann GmbH, Frankfurt, Germany (design), Coyne et Bellier, Lyon, France (execution); VS_A, Lille, France (execution)

HVAC: ITEE-Fluides, Arnas, France

Security Fire Consultation: Cabinet Casso & Associés, Paris, France

Acoustics: Cabinet Lamoureux, Paris, France

Media Consulation: Cabinet Labeyrie, Paris, France

Lighting Consultation: Har Hollands, Eindhoven, Netherlands

Landscape Design: EGIS aménagement, Lyon, France

www.coop-himmelblau.at